## PLANTING SEA OATS PERPENDICULAR TO THE SHORELINE IS PROHIBITED.

Recommended supplies:

Tape measure (preferably 100’ or more)

Wooden or PVC stakes (approx. 4’ long) for marking off planting area flagging tape

Yard sticks, tape measures or pieces of flagging tape cut to 1.5 - 2 ft. lengths Spades/hand trowels/broom handles/PVC pipe (plan on 2 shovels for every 3 volunteers)

## Steps for planting:

- Select the area along the seawall to be planted and rope off three sides (along the top of the dune and down two sides, leaving the beach front open).

- Run a tape measure along the edge of the seawall/landward extend of planting area and mark off a point every 1 ft by making a depression in the sand. Sea oats rows must be parallel to the shoreline.

Stagger rows to start and end at the mid point of the previous row:

SEAWALL/LANDWARD EDGE	SEAWALL/LANDWARD EDGE

1 ft Spacing

6 in offset 1 ft Spacing

1 ft Spacing

2 ft Spacing

1 ft offset 2 ft Spacing

2 ft Spacing

4 ft spacing 1 gal plants

# ●     ●     ●     ●     ●     ●     ●     ●     ●     ●     ●     ●     ●

●     ●     ●     ●     ●     ●     ●     ●     ●     ●     ●     ●

●     ●     ●     ●     ●     ●     ●     ●     ●     ●     ●     ●     ●

●            ●             ●             ●            ●            ●             ●

●            ●            ●            ●            ●            ●

●            ●            ●             ●            ●            ●             ●

●               ●                ●                 ●

- Dig to make a hole 12" deep and at least 1-1/2" in diameter at each marked point.

- Place one sea oat seedling in each hole, making sure that the top of the root bulb is at least 8" below the surface of the sand (failure to plant deep enough is the primary cause of plant death). Fill in the hole around the seedling with sand.

- If desired, work one teaspoon of time-release fertilizer into sand around the base of each plant.

- Repeat steps 2-5 for additional rows of sea oats, making sure that each row is staggered and 1’ apart. This can be done using a long tape measure or with a 1’ long marker. After approximately half of the rows are marked change the spacing to 2 ft.  The final most seaward row will consist of gallon plants (if provided) spaced 4 ft apart.

- Plants should be watered immediately following planting, then again weekly for the first two months. Follow the water restrictions of your community.

- Mark off the planted area with small stakes and flagging tape to avoid pedestrian impacts.  Please contact Charles Grisafi Charles.Grisafi@dep.state.fl.us with DEP for more information on the types of signage allowed.

References

Rogers, Spencer, and David Nash. The Dune Book. 2003, https://ncseagrant.ncsu.edu/ncseagrant_docs/products/2000s/dune_book.pdf

UF/IFAS Seagrant Dune Restoration and Enhancement for the Florida Panhandle Manual:

http://edis.ifas.ufl.edu/sg156